# Magic Leap 2 AprilTag Spatial Accuracy Measurement

## Purpose and Value

This project demonstrates how to quantitatively measure spatial accuracy and stability of an augmented-reality (AR) headset — specifically the Magic Leap 2 — using AprilTags as known physical reference points.

It answers two key questions:
1. How accurately does the headset align virtual content with real-world geometry?
2. How stable is that alignment over time as the headset’s tracking system updates its world model (drift)?

This capability is highly valuable in shipbuilding, industrial alignment, and digital-twin workflows, where millimeter-level precision determines whether virtual overlays align correctly with physical structures.

## What We Built

We created a Unity-based application that:
• Detects AprilTags placed in the real world through the Magic Leap 2 camera.
• Calculates the 3D pose (position and orientation) of each detected tag relative to the headset.
• Places a virtual object (such as a cube or hologram) at the corresponding position in Unity’s world space.
• Continuously measures positional and rotational drift between successive detections.
• Displays real-time data (position, rotation, drift) on a HUD overlay visible through the headset.

## How We Achieved It

1. Environment Setup
   • Built with Unity 6 and the Universal Render Pipeline (URP), deployed to Magic Leap 2 via OpenXR.
   • Enabled Magic Leap Rendering Extensions with Additive blend mode to achieve transparent passthrough.

2. AprilTag Detection
   • Used a custom detector (MagicLeapTagDetector_260) to process camera frames and trigger TagObservation events (~1 Hz).
   • Each observation provided the AprilTag’s unique ID and its world-space pose.

3. TagPlacementController
   • Subscribed to tag detection events.
   • Computed where the virtual object should appear relative to each detected tag.
   • Calculated positional and angular drift between current and intended object poses.
   • Updated a TextMeshPro-based HUD displaying tag ID, pose, and drift values.

4. Visualization
   • Built a bottom-aligned HUD Canvas (Screen Space – Camera, UI layer) rendered over passthrough video.
   • Verified visibility and transparency using Magic Leap’s additive rendering mode.

## Outcome

• Developed a functional AR testbed that quantifies the spatial alignment performance of the Magic Leap 2.
• Provides a tool for measuring both accuracy and drift of virtual-to-physical alignment.
• Enables empirical comparison between different AR devices and tracking configurations.
• Establishes a foundation for future calibration experiments involving multiple tags, spatial anchors, and heatmaps of drift.

In summary, this project built a precise AR measurement framework that uses AprilTags as physical reference markers and leverages Unity and Magic Leap 2 to visualize and quantify how accurately and consistently the headset maintains alignment between the virtual and real worlds.